Resources:

Nvidia: https://developer.nvidia.com/docs/drive/drive-os/6.0.5/public/driveworks-nvsdk/dwx_camera_calibration.html

Camera Calibration Guide

This guide shows how to calibrate one or more cameras and find the camera values saved by the script.

# What you will get

The script can create two types of results:

- Intrinsic values: the camera matrix and lens distortion. Use these values to remove lens distortion or do camera geometry.

- Extrinsic values: the camera position and direction compared with a vehicle. This part is optional.

If you only need to remove lens distortion, do the intrinsic calibration and skip the extrinsic calibration.

# Step 1: Check your checkerboard

The script is currently set to:

CHECKERBOARD_INNER_CORNERS = (8, 6)

SQUARE_SIZE_METERS = 0.025

This means your checkerboard must have:

- 8 inside corners across

- 6 inside corners down

- 9 squares across and 7 squares down

- Squares that are 25 mm wide

Count the inside corner points, not the squares. Measure one square carefully. If your squares are 30 mm wide, change SQUARE_SIZE_METERS to 0.030.

Keep the checkerboard flat. Glue or tape it to a hard, flat board. A bent checkerboard can give bad results.

# Step 2: Check the script settings

Open camera_calibration.py and look at the settings near the top:

IMAGES_PER_CAMERA = 20

INITIAL_DELAY_SECONDS = 30

CAPTURE_INTERVAL_SECONDS = 7

CHECKERBOARD_INNER_CORNERS = (8, 6)

SQUARE_SIZE_METERS = 0.025

FIRST_OUTPUT_CAMERA_NUMBER = 4

REQUESTED_WIDTH = 1920

REQUESTED_HEIGHT = 1080

MINIMUM_VALID_IMAGES = 10

The script takes 20 pictures. It waits 30 seconds before the first picture, then takes one picture every 7 seconds. At least 10 pictures must show the full checkerboard.

The first detected camera is named Camera 4. If you want it to be Camera 1, change this line:

FIRST_OUTPUT_CAMERA_NUMBER = 1

# Step 3: Install Python packages

Open a terminal in the folder that contains camera_calibration.py. You can also use cd to go to that folder. Replace the example path below with the real folder on your computer.

cd path/to/Calib

Create a Python environment and install the required packages:

python3 -m venv .venv

source .venv/bin/activate

python3 -m pip install opencv-python numpy

On Windows Command Prompt, activate the environment with:

.venv\Scripts\activate

If your computer uses python instead of python3, use python in the commands.

# Step 4: Prepare the camera

- Connect the camera and close apps that may be using it.

- Set the focus, zoom, and resolution you plan to use later.

- Turn off autofocus after focusing, if possible.

- Fix the camera in place. Do not move it during calibration.

Do not change the focus, zoom, or resolution after calibration. If you change them, calibrate the camera again.

# Step 5: Run the script

From the folder that contains camera_calibration.py, run:

python3 camera_calibration.py

The script searches for connected cameras. It may print something like:

Detected Camera 1 -> Calibration Camera 4 -> device index 0

This means OpenCV found device 0 and the output files will call it Camera 4. Type C in the terminal to continue.

# Step 6: Check the preview

The preview window uses these keys:

- C: start taking calibration pictures

- R: test the image brightness

- S: skip this camera

- Q or Escape: stop the program

Make sure the preview shows the correct camera. Press R to test brightness. Also look at the image yourself. The checkerboard should be bright enough, sharp, and free from strong reflections.

# Step 7: Take good calibration pictures

Press C. The script waits before taking the first picture. Keep the camera still and move the checkerboard to a new position after every picture.

Use a good mix of views:

- Put the board near the center in a few pictures.

- Put it near the left, right, top, bottom, and all four corners.

- Move it closer and farther away.

- Tilt and rotate it in different directions.

- Keep the complete checkerboard inside the image.

Do not use blurry pictures. Do not hide or cut off part of the board. Do not take all pictures from the same position.

Pictures are saved in a folder like calibration_images/camera_4. Before you start a new final run, move or delete old pictures from that camera folder so old files are not used by mistake.

# Step 8: Get the intrinsic values

After taking the pictures, the script looks for the checkerboard in each image. If it finds the board in at least 10 images, it calculates the calibration.

The results are saved in the calibration_results folder:

camera_4_intrinsics.json

camera_4_intrinsics.npz

Open the JSON file to read the values. The most important values are:

- fx and fy: focal length in pixels

- cx and cy: the center point of the camera image

- camera_matrix: the full 3 x 3 camera matrix

- distortion_coefficients: the lens distortion values

- rms_error: a measure of calibration quality

The camera matrix looks like this:

[[fx,  0, cx],

[ 0, fy, cy],

[ 0,  0,  1]]

For the normal OpenCV model, the distortion values are usually in this order:

[k1, k2, p1, p2, k3]

# Step 9: Check the result

A smaller RMS error is usually better:

- Below about 0.5 pixels: very good

- About 0.5 to 1.0 pixels: often usable

- Above about 1.0 pixels: check your pictures and try again

- Several pixels: the calibration is probably poor

These are general guidelines, not strict rules. Also test the values by removing distortion from an image. Straight lines should look straight, especially near the image edges.

The script also prints mean_reprojection_error_pixels. Its formula is different from the common per-corner mean, so it may look unusually small. Use the RMS error as the main number and use the mean value to compare runs made with this same script.

# Step 10: Load the saved values

Use this Python code to load the NPZ file:

import numpy as np

data = np.load("calibration_results/camera_4_intrinsics.npz")

camera_matrix = data["camera_matrix"]

distortion = data["distortion_coefficients"]

image_size = data["image_size"]

print(camera_matrix)

print(distortion.ravel())

print(image_size)

Change camera_4 to the camera number created by your run.

# Step 11: Remove lens distortion

import cv2

import numpy as np

data = np.load("calibration_results/camera_4_intrinsics.npz")

K = data["camera_matrix"]

dist = data["distortion_coefficients"]

image = cv2.imread("test_image.png")

h, w = image.shape[:2]

new_K, roi = cv2.getOptimalNewCameraMatrix(K, dist, (w, h), 0, (w, h))

fixed = cv2.undistort(image, K, dist, None, new_K)

x, y, rw, rh = roi

if rw > 0 and rh > 0:

fixed = fixed[y:y+rh, x:x+rw]

cv2.imwrite("test_image_undistorted.png", fixed)

Use an image with the same resolution, focus, and zoom used during calibration.

# Optional: Calibrate the camera position

After the intrinsic calibration, type E to do extrinsic calibration or S to skip it.

The vehicle directions are:

+X = forward

+Y = left

+Z = up

Keep the camera fixed. Put the checkerboard at a position you can measure from the vehicle origin. The script marks one inside corner as O and shows the board +X and +Y directions.

## How to enter x0, y0, and z0

These three numbers are the position of the checkerboard corner marked O. They are not the camera position.

First choose one fixed point on the vehicle as the vehicle origin. Measure from that point to the exact inside checkerboard corner marked O in the preview.

- x0: how far O is forward from the vehicle origin

- y0: how far O is left from the vehicle origin

- z0: how far O is above the vehicle origin

Enter every value in meters. Use a negative number when O is behind, to the right, or below the vehicle origin.

## Example

Suppose O is 1.50 m forward, 0.40 m to the right, and 0.60 m above the vehicle origin. Enter:

x0 (forward): 1.50

y0 (left): -0.40

z0 (up): 0.60

The right-side measurement is negative because +Y points left. Measure the marked O corner carefully. Wrong measurements or signs will give a wrong camera-to-vehicle transform, even if the reprojection image looks good.

- Measure the exact marked O corner in meters.

- Enter its forward, left, and up position.

- Tell the script which vehicle direction the board +X and +Y arrows point.

Right, backward, and down are negative values. The board axes must line up with vehicle directions and must be at right angles. If the board is at a random angle, this script cannot describe that angle correctly.

The script saves a camera-to-vehicle rotation, translation, 4 x 4 transform, viewing direction, and a reprojection image. In the reprojection image, the red circles and green crosses should almost overlap.

# Common problems

No camera found: Check camera permission, the cable, and whether another app is using the camera.

No preview window: Install opencv-python, not only opencv-python-headless. Run the script on a computer with a desktop screen.

Board not found: Check the inside-corner count. Improve focus and lighting. Make sure the whole board is visible.

Not enough valid images: Take more clear pictures with the complete board visible.

Bad values: Check the corner count, board flatness, camera settings, image resolution, and variety of board positions.

Fisheye camera: This script uses a normal camera model. A strong fisheye lens may need OpenCV's fisheye calibration tools.

# Final checklist

- The inside-corner count is correct.

- The square size is measured correctly.

- The board is flat and the images are sharp.

- The camera does not move during calibration.

- The pictures cover the center, edges, corners, distances, and angles.

- At least 10 images are valid; 15 to 20 good images are better.

- The RMS error is reasonable.

- An undistorted test image looks correct.

- The JSON and NPZ files are saved with the camera setup.